SDN IP BGP ONOS Docummentation:

Topology:

Devices:

1. PC Server with OS Ubuntu Server 16.04 LTS ( ONOS Controller + Speaker installed)

2. Linux Quagga Router

3. Cisco Router 2811

4. Pica8 Switch

Technical Allocation:

| No | Device | Interface | IP address | AS number |
| --- | --- | --- | --- | --- |
| 1 | PC Server (ONOS+Speaker) | Management | 10.10.11.11/20 |  |
|  |  | Enp4s2.1 | 172.16.1.1/24 | 65000 |
|  |  | Enp4s2.2 | 172.16.2.1/24 | 65000 |
| 2 | Linux Quagga Router | Management | 10.10.11.12/20 |  |
|  |  | Ens8 | 172.16.1.2/24 | 65001 |
| 3. | Cisco Router 2811 | Management | 10.10.11.13/20 |  |
|  |  | Fast Ethernet 0/1 | 172.16.2.2/24 | 65002 |
| 4. | Pica8 Switch | 1/1/1 |  |  |
|  |  | 1/1/2 |  |  |
|  |  | 1/1/3 |  |  |

Configuration:

1. PC Server

- Configure PC Server interface so it can have 2 subinterface in single interface:

ip link set up dev enp4s2

ip link add link enp4s2 dev enp4s2-1 address 00:00:00:00:00:a1 type macvlan

ip link add link enp4s2 dev enp4s2-2 address 00:00:00:00:00:a2 type macvlan

- Configure IP address for each subinterface

ifconfig enp4s2-1 172.16.1.1 netmask 255.255.255.0 up

ifconfig enp4s2-2 172.16.2.1 netmask 255.255.255.0 up

2. PC Server (ONOS)

-  Install ONOS controller from (https://wiki.onosproject.org/display/ONOS/Downloads)

-  Configure ONOS BGP Peering to Speaker Router, Quagga Linux Router, and Cisco Router 	2911 (network-cfg.json)

{

"ports" : {

"of:0000000000000001/1" : {

"interfaces" : [

{

"ips"  : [ "172.16.1.1/24" ],

"mac"  : "52:54:00:ec:4c:59"

},

"of:0000000000000001/2" : {

"interfaces" : [

{

"ips"  : [ "172.16.2.1/24" ],

"mac"  : "00:14:6a:a3:08:11"

},

"apps" : {

"org.onosproject.router" : {

"bgp" : {

"bgpSpeakers" : [

{

"name" : "speaker1",

"connectPoint" : "of:0000000000000001/3",

"peers" : [

"172.16.1.2",

"172.16.2.2"

3.  PC Server (Speaker)

- Install Quagga Router in PC Server as a Speaker Router

- Configure BGP Speaker Router with to peering to ONOS (ASN 65000), Linux Quagga Router 	(ASN 65001) , and Cisco Router 2811 (ASN 65002)

router bgp 65000

bgp router-id 10.10.11.11

timers bgp 3 9

! ONOS SDN IP

neighbor 127.0.0.1 remote-as 65000

neighbor 127.0.0.1 port 2000

neighbor 127.0.0.1 timers connect 5

! BGP External 1

neighbor 172.16.1.2 remote-as 65001

neighbor 172.16.1.2 ebgp-multihop 255

neighbor 172.16.1.2 advertisement-interval 5

neighbor 172.16.1.2 timers connect 5

!  BGP External 2

neighbor 172.16.2.2 remote-as 65002

neighbor 172.16.2.2 ebgp-multihop 255

neighbor 172.16.2.2 advertisement-interval 5

4.  Linux Router (Quagga)

- Install Quagga Router as Linux Router

- Configure BGP Quagga Router to peering to Speaker Router (ASN 65000)

router bgp 65001

bgp router-id 10.10.11.12

timers bgp 3 9

neighbor 172.16.1.1 remote-as 65000

neighbor 172.16.1.1 ebgp-multihop

neighbor 172.16.1.1 timers connect 5

neighbor 172.16.1.1 advertisement-interval 5

5.  Cisco Router 2811

- Configure BGP to peering to Speaker Router (ASN 65000)

router bgp 65000

bgp router-id 10.10.11.13

timers bgp 3 9

neighbor 172.16.2.1 remote-as 65000

neighbor 172.16.2.1 ebgp-multihop

neighbor 172.16.2.1 timers 5

neighbor 172.16.2.1 advertisement-interval 5

4. Pica8 Switch

-  set controller pica8switch to ONOS Controller

ovs-vsctl set-controller br0 tcp:10.10.11.11:6633

BGP Route Prefix Inject Prerequisite: (in Linux Quagga Router)

1. In Linux Quagga Router, install BGPDump command with apt or download binary data from (http://www.ris.ripe.net/source/)

2. Get dumps route information from  (http://data.ris.ripe.net/rrc00/), use latest-bview.gz.

3. Compile latest bview using bgp dump command ,

“zcat latest-bview.gz | bgpdump -m - > myroutes”

4. From myroutes file we will got around 2 millions router prefix,, filter duplicate rows with Awk Command,

“awk -F'|' '!seen[$6]++' myroutes >> myroutes”

5. Download BGP Simple Script for push/inject route prefix information from (https://github.com/xdel/bgpsimple), download “bgp_simple.pl”

BGP Route Prefix Inject Tutorial:

1. in Linux Quagga Router:

- inject router prefix from “myroute” file with bgp_simple script :

“sudo ./bgp_simple.pl “option” “

: -myas <own AS number>

: -myip <own IP Address>

: -peerip <neighbor IP Address>

: -peeras <neighbor AS Number>

: -p <push route information dump location>

: -m <number of routes>

: -v <verbose>

: -keepalive 5 -holdtime 20 <timers>

2. Check BGP prefix in Speaker Router with command :

“ show ip bgp summary”

3. Check BGP prefix in Cisco Router 2911 with command :

“ show ip bgp summary”

4. Check BGP Prefix in ONOS Controller with command :

“ bgp-routes”

RESULT:

1.  Push/inject 1000 routes prefix :

- command 1000 routes :

“sudo ./bgp_simple.pl -myas 65001 -myip 172.16.1.2 -peerip 172.16.1.1 -peeras 65000 -p 	/home/user/myroutes -m 1000 -v -keepalive 5 -holdtime 20”

- result 1000 routes :

3.  Push/inject 10000 routes prefix :

- command 10000 routes :

“sudo ./bgp_simple.pl -myas 65001 -myip 172.16.1.2 -peerip 172.16.1.1 -peeras 65000 -p 	/home/user/myroutes -m 10000 -v -keepalive 5 -holdtime 20”

- result 10000 routes :

4.  Push/inject 20000 routes prefix :

- command :

“sudo ./bgp_simple.pl -myas 65001 -myip 172.16.1.2 -peerip 172.16.1.1 -peeras 65000 -p 	/home/user/myroutes -m 20000 -v -keepalive 5 -holdtime 20”

- result 20000 routes :